ECE-111 Advanced Digital Design Projects

Homework-3a:

Develop SystemVerilog RTL model for 4-bit Johnson Counter and perform following mentioned :

- Synthesize Johnson counter

- Run simulation using Johnson counter testbench provided

- Review synthesis results (resource usage and RTL netlist/schematic)

  - Note : Explanation of resource usage in report is not mandatory to provide.

- Review input and output signals in simulation waveform.

About Johnson Counter :

- Similar to Straight Ring counter with below mentioned difference

- Johnson counter connects the complement of the output of the last shift register to the input of the first register and circulates a stream of ones followed by zeros around the ring

- Johnson Counter has 2N states (if N is 4, then total states for counting is 8. And 4-bit pattern will repeat after 8 cycles).

4-bit Johnson Counter with initial load count value = 4’b0000

4-bit Johnson Counter with initial load count value = 4’b0000

4-bit pattern repeats every 8 cycles

4-bit pattern repeats every 8 cycles

HW-3b:

Report the output sequence if we preload with one of the 8 remaining states not covered by preloading with 0000.

Homework Submission :

Submit report (PDF file) which should include:

- Snippet of SystemVerilog design code

- RTL netlist viewer schematic and Synthesis resource usage

  - Optional: Resource usage explanation and post mapping schematic

- Simulation snapshot and explain simulation result to confirm it works as a Johnson counter

- For part 3-b, the sequence of count[3:0] values, in binary.

Note:

- When creating RTL model for Johnson Counter, name SystemVerilog Module name as : johnson_counter

- Assume below mentioned Primary Ports for Johnson Counter

  - Input clk (clock)

  - Input clear (asynchronous reset / negedge signal)

  - Input preset (synchronous and active low signal)

  - Input load_cnt (value gets loaded when !preset)

  - Output  count (output count value)

Johnson Counter

clk

clear

count[0]

count[1]

count[2]

count[3]

preset

load_cnt

Johnson Counter

clk

clear

count[0]

count[1]

count[2]

count[3]

preset

load_cnt

4-bit pattern repeats every 8 cycles

4-bit pattern repeats every 8 cycles

Lab3.zip folder posted on Piazza and Canvas has johnson_counter_testbench.sv testbench code to simulate with johnson counter SystemVerilog module.

Note :

- 3-a: Assume initial load_cnt signal value is set to 4’b0000 in testbench code provided.

- 3-b: Students update johnson_counter_testbench.sv code for initial load_cnt value !0.