ECE-111 Advanced Digital Design Projects

Homework-8

Design and Develop SystemVerilog FSM code for Uart Receiver (Moore implementation)

- Develop a Moore FSM implementation of UART receiver (Rx)

- Develop state transition diagram. Use binary state encoding for states

- Assume UART Tx module will send 8-bit serial data to Uart Rx without parity bit

- Start bit (negedge) will be send before transmitting 8-bit serial data

- Stop-bit (posedge) will be send after transmitting 8-bit serial data

- Uart Rx will convert 8-bit serial data (rx) and generate 8-bit parallel data (rx_dout)

- Rx_dout will be set to ‘1’ when 8-bit rx_dout parallel data is available otherwise set to ‘0’

- Baud rate requirement for Uart-Rx and Uart-Tx is 115200 bps. Which means set NUM_CLKS_PER_BIT=434 for Uart Tx and Rx clock period = 20ns. However for faster simulation results design Uart Rx with NUM_CLKS_PER_BIT=16

- Assume 16 clock cycle sampling for each incoming serial input bit to UART-Rx (rx_clk) (this is same as setting parameter NUM_CLKS_PER_BIT=16)

- Assume rx_rstn is synchronous active low reset

DELIVERABLES

- Develop state transition diagram and  state transition table for the implementation (Moore)

- Use my testbench for uart-rx receiver and simulate FSM to confirm its behavior. Review waveform and provide explanation in report

- Review resource utilization and provide total flipflops and combination ALUTs in report

- Provide Uart Rx RTL code, synthesis netlist, state machine diagram or state transition table, simulation waveform snapshot, explanation of simulation result and simulation transcript (prints from testbench) in homework report.

- Primary port list :

- Input clk (clock)

- Input rstn (synchronous reset / negedge signal)

- Input rx (8-bit input serial input bit stream + 1 bit start and 1 bit stop bit)

- Output done (1-bit output. ‘1’ when 8-bit dout is available)

- Output dout (8-bit parallel data output)

- Your design top systemverilog module name should be named as uart_rx and filename should be uart_rx.sv